# Arm

# Inverse Kinematics:

2 DOF, 3 DOF,

cos(θ) = (x² + y² - L₁² - L₂²) / (2L₁L₂)

6 DOF:

Equation: Wc = P – d₆ · Rz

Where:

- Wc: Wrist Centre

- P: Desired gripper position

- d₆: Length of gripper

- Rz: Orientation described by rotation matrix

Dimensions of arm:

Let the arm be attached at 45 cm of height from base.

Spearhead Rack 		Rod Rack

Height of spear to be picked up = 50cm + 5.25cm = 55.25cm

Link1 = 10cm, Link2 = 22.5cm, Link3 = 22.5cm, Link4 = 5cm,

Link5 = 5cm, Link6 = 5cm

Total = 60cm arm length

Reach Calculations:

- Lowest height of reach = 55.25 cm

- Highest height of reach = 100 cm

[ Considering 7003cm3 volume.]

Shoulder torque:

Equation: τ = (Larm × Wobj) + (Lcg× Warm)

Where:

- τ: Torque

- Larm: Length of arm

- Wobj: Weight of object

- Lcg: Length to Center of Gravity (CG)

- Warm: Weight of arm

Factor of Safety (FOS):

- Load capacity considered = 500 gram

Material:

- Arm made of aluminium / plastic

Weight Distribution (Total arm weight = 1 kg):

- Link 1 — 250 g

- Link 2 — 300 g

- Link 3 — 250 g

- Gripper — 200 g

J3 Calculation(Forearm, wrist & payload):

Assumption: Max moment is calculated, thinking arm is horizontal.

Equation: τJ3 = [m₃ × (l₃ / 2)] + (mw × l₃) + (mp × [l₃ + lw])

Substitution: τJ3 = [0.25 × 11.25] + (0.2 × 22.5) + (0.5 × 27.5)

Result: τJ3 = 21.06 kg-cm

J2 Calculation:

Equation: τJ2 = ML2 + ML3 + M W + M payload

Result: τ J2 = 45.81 kg-cm

Dynamic Torque:

Formula: τTotal = τstatic + variable

Calculations:

- J2: 45.81 × 2 = 91 kg-cm

- J3: 21.06 × 2 = 42 kg-cm

- J4 - J6: 15 kg-cm (Wrist)

For base,

Parameters:

- α = 5.23 rad/s² (approx)

- m = 1.5 kg

- max reach = 50 cm

Torque Calculations:

- τinertia = 0.653 N·m

- τfrictional  = 0.15 N·m

Base Torque:

- τbase  = 8.19 kg-cm × 3

Final Result:  τ = 24.57 kg-cm